UNIVERSAL RT SCAN ANALYSIS

LLM INTEGRATION BUILD SHEET v2.0

Any Part. Any Geometry. Any Code.

Two-Stage LLM Pipeline | Geometry Classification | Adaptive Prompting

CHANGES FROM v1.0:

Single-prompt → Two-stage pipeline (classify → analyze)

Hardcoded vessel geometry → Universal geometry schema

Fixed Three.js primitives → Geometry factory from LLM output

ASME VIII only → Multi-code support (VIII, B31.3, D1.1, AMS, etc.)

Onnex AI Agency  |  Las Vegas, NV  |  March 2026  |  v2.0

# 1. WHY THE v1.0 SINGLE-PROMPT APPROACH BREAKS

The v1.0 build sheet assumed every input is a pressure vessel. The system prompt was hardcoded with ASME VIII weld categories (A/B/C/D per UW-3), cylindrical geometry assumptions, and vessel-specific defect patterns. When you drop in a pipe spool, a turbine disk, a structural weldment, or a casting, three things break:

- Geometry mismatch — The renderer assumes a cylinder with elliptical heads. A pipe elbow, a flange-to-reducer transition, or a flat plate weldment can’t be represented by that parametric model.

- Code mismatch — Not everything is ASME VIII. Piping follows B31.3. Structural steel follows AWS D1.1. Aerospace castings follow AMS specs. The system prompt cited the wrong acceptance criteria.

- Defect profile mismatch — A forged turbine disk has different defect expectations (inclusions, hydrogen flaking, grain flow anomalies) than a rolled-plate welded vessel (lack of fusion, slag, porosity).

The fix is a two-stage pipeline. Stage 1 classifies the part and extracts geometry. Stage 2 runs the RT analysis with a dynamically assembled prompt tuned to what Stage 1 identified.

# 2. TWO-STAGE PIPELINE ARCHITECTURE

## 2.1 Pipeline Flow

| Step | Stage | Action | Output |
| --- | --- | --- | --- |
| 1 | Ingestion | Upload drawing (PDF/DXF/image/STEP). OCR + layout extraction. | Raw extracted text + region map |
| 2 | ITAR Check | ndtv1-comply scans for ITAR/EAR markings. Sets routing flag. | useLocal: boolean |
| 3 | STAGE 1: Classify | LLM identifies part type, geometry primitives, applicable code, material class, weld/feature map. | PartClassification JSON |
| 4 | Prompt Assembly | Select code-specific analysis module based on Stage 1 output. Build composite system prompt. | Assembled system + user prompt |
| 5 | STAGE 2: Analyze | LLM performs RT analysis with part-specific context. Outputs overlay zones. | RTAnalysis JSON |
| 6 | Validate | Schema validation, code-reference check, severity monotonicity, position bounds. | Validated analysis or retry |
| 7 | Render | Geometry factory builds 3D model from classification. Overlay factory maps zones. | Interactive 3D + overlays |
| 8 | Report | Generate shot plan, compliance report, downloadable DOCX/PDF. | Inspection report |

## 2.2 Supported Part Types

| Part Type | Geometry Primitives | Primary Code | RT Considerations |
| --- | --- | --- | --- |
| Pressure Vessel | cylinder, elliptical_head, hemisphere, cone, torus | ASME VIII Div.1/2 | UW-11 joint categories, full/spot RT |
| Pipe Spool | cylinder, elbow, tee, reducer, flange | ASME B31.3 / B31.1 | Random RT %, wall-thickness-dependent technique |
| Structural Weldment | plate, beam, gusset, stiffener | AWS D1.1 / D1.6 | CJP vs PJP weld classification |
| Casting | freeform_solid, core_passage, boss | ASTM E446 / E186 / E280 | Reference radiograph comparison, shrinkage/porosity grading |
| Forging | disk, ring, shaft, block | ASTM E1030 / AMS specs | Grain flow, inclusion detection, UT typically primary |
| Aerospace Component | airfoil, skin_panel, spar, rib, fitting | NAS 410 / Nadcap AC7114 | Digital radiography, computed tomography for complex geometry |
| Tube/Heat Exchanger | tubesheet, tube, baffle, shell | ASME VIII + TEMA | Tube-to-tubesheet weld RT, miniature crawlers |
| Tank (Storage) | cylinder, flat_bottom, cone_roof, dome_roof | API 650 / 620 | Shell course butt welds, annular plate joints |

# 3. STAGE 1: CLASSIFICATION PROMPT

Stage 1 runs first on every uploaded drawing. Its job is to identify what the part is, extract its geometry into a universal schema, and determine which code and analysis module to use. This prompt never changes — it’s the universal entry point.

## 3.1 Stage 1 System Prompt

You are an engineering drawing analyst specializing in NDT-inspectable

parts across all industries: pressure equipment, piping, structural

steel, castings, forgings, and aerospace components.

YOUR TASK:

Analyze the provided drawing data and classify the part for

radiographic testing (RT) inspection planning.

You MUST determine:

1. PART TYPE (exactly one):

pressure_vessel | pipe_spool | structural_weldment | casting |

forging | aerospace_component | heat_exchanger | storage_tank |

other

2. APPLICABLE CODE (primary + supplementary):

ASME_VIII_1 | ASME_VIII_2 | ASME_B31.3 | ASME_B31.1 |

AWS_D1.1 | AWS_D1.6 | API_650 | API_620 | ASTM_E446 |

ASTM_E186 | ASTM_E1030 | NAS_410 | Nadcap_AC7114 |

EN_13445 | EN_12952 | PED_2014/68/EU | other

3. GEOMETRY DECOMPOSITION:

Break the part into primitive shapes from this set:

cylinder | cone | sphere | hemisphere | elliptical_head |

torus_segment | elbow | plate | beam | disk | ring | shaft |

tubesheet | airfoil | skin_panel | freeform_solid | flange |

reducer | nozzle | saddle | skirt | lug | gusset | stiffener |

boss | pad | doubler

Each primitive needs: dimensions, position/orientation relative

to a common origin, and material.

4. FEATURE MAP:

Identify all inspectable features:

butt_weld | fillet_weld | socket_weld | seal_weld | tack_weld |

braze_joint | bolt_hole_pattern | machined_surface | bore |

keyway | spline | thread | casting_junction | parting_line |

core_print | riser_location | gate_location | grain_flow_line |

heat_affected_zone | clad_interface | overlay_weld | repair_weld

5. MATERIAL CLASSIFICATION:

Identify material spec, type (rolled_plate, forged, cast,

extruded, wrought), and any special metallurgical concerns

(hydrogen embrittlement, sensitization, sigma phase, etc.)

OUTPUT: Respond with ONLY valid JSON, no markdown fences.

## 3.2 Stage 1 User Prompt Template

Classify the following engineering drawing for RT inspection planning.

<drawing_data>

<raw_text>{ocr_extracted_text}</raw_text>

<title_block>

<drawing_number>{drawing_number}</drawing_number>

<revision>{revision}</revision>

<title>{title}</title>

<material_spec>{material_callout}</material_spec>

<code_stamp>{code_stamp_if_visible}</code_stamp>

</title_block>

<bill_of_materials>{bom_json_if_extracted}</bill_of_materials>

<notes>{drawing_notes_text}</notes>

<weld_symbols>{extracted_weld_symbols_json}</weld_symbols>

<dimensions>{key_dimensions_json}</dimensions>

</drawing_data>

Respond with ONLY valid JSON matching this schema:

{

"part_id": "string (drawing number or serial)",

"part_type": "string (from allowed list)",

"part_description": "string (1-2 sentence summary)",

"applicable_codes": {

"primary": "string",

"supplementary": ["string"]

},

"geometry": {

"bounding_box": {

"length": { "value": 0, "unit": "in|mm" },

"width": { "value": 0, "unit": "in|mm" },

"height": { "value": 0, "unit": "in|mm" }

},

"primitives": [

{

"id": "string (e.g. SHELL-1, HEAD-FWD, ELBOW-1)",

"type": "string (from allowed primitives)",

"dimensions": {

"od": 0, "id": 0, "length": 0, "thickness": 0,

"angle": 0, "radius": 0,

"unit": "in|mm"

},

"position": {

"x": 0, "y": 0, "z": 0,

"rotation_deg": { "rx": 0, "ry": 0, "rz": 0 }

},

"material": "string"

}

]

},

"features": [

{

"id": "string (e.g. LS-1, GS-3, FW-2, BH-1)",

"type": "string (from allowed features)",

"connects": ["primitive_id_a", "primitive_id_b"],

"weld_type": "CJP|PJP|fillet|socket|seal|NA",

"location_description": "string",

"position": {

"x_normalized": 0.0, "y_normalized": 0.0,

"z_normalized": 0.0, "angle_degrees": 0

}

}

],

"materials": [

{

"spec": "string (e.g. SA-516-70)",

"form": "rolled_plate|forged|cast|extruded|wrought",

"applied_to": ["primitive_id"],

"metallurgical_concerns": ["string"]

}

],

"design_conditions": {

"pressure": { "value": 0, "unit": "psi|bar|MPa" },

"temperature": { "value": 0, "unit": "F|C" },

"service": "string (general|lethal|hydrogen|cyclic|cryogenic)",

"pwht_required": true,

"impact_test_required": true,

"additional_notes": "string"

},

"rt_requirements_from_drawing": {

"stated_rt_extent": "string (raw text from drawing notes)",

"acceptance_standard": "string (if stated)"

},

"analysis_module": "string (see mapping below)",

"confidence": 0.0-1.0

}

ANALYSIS_MODULE MAPPING:

pressure_vessel    -> "asme_viii_vessel"

pipe_spool         -> "asme_b31_piping"

structural_weldment -> "aws_structural"

casting            -> "casting_radiography"

forging            -> "forging_inspection"

aerospace_component -> "aerospace_ndt"

heat_exchanger     -> "heat_exchanger"

storage_tank       -> "api_tank"

other              -> "generic_rt"

# 4. STAGE 2: ANALYSIS PROMPT (MODULAR)

Stage 2 receives the Stage 1 classification output and uses it to assemble a part-specific analysis prompt. The system prompt is composed of a BASE module (always included) plus a CODE-SPECIFIC module selected by the analysis_module field from Stage 1.

## 4.1 Base System Prompt (Always Included)

You are an NDT Level III radiographic testing analyst with

multi-code, multi-industry expertise.

You have been provided with:

1. A part classification from the first analysis stage

2. A code-specific analysis module (appended below)

UNIVERSAL ANALYSIS FRAMEWORK:

For ANY part type, you MUST:

A) IDENTIFY all inspectable zones and classify by severity:

CRITICAL - Highest defect probability, structural failure risk,

or code-mandated full examination

HIGH     - Full-penetration welds, high-stress regions,

material transition zones

MEDIUM   - Partial exam welds, lower-stress attachments,

secondary structural members

LOW      - Non-pressure-boundary, cosmetic, or non-structural

B) MAP each zone to a 3D position using normalized coordinates:

x_normalized: 0.0 (start/left/front) to 1.0 (end/right/rear)

y_normalized: 0.0 (bottom) to 1.0 (top)

z_normalized: 0.0 (near) to 1.0 (far)

angle_degrees: 0-360 for circumferential features

span_degrees: angular extent of the zone (0 for point features)

For features that wrap around a primitive (welds on cylinders),

use angle_degrees for the start position and set

span_degrees = 360 for full circumferential.

C) SPECIFY expected defects with probability and code references:

Each defect must cite the specific code paragraph governing

its acceptance/rejection criteria.

D) RECOMMEND RT technique per zone:

Source, SFD, film/DR, IQI type and placement, technique

(SWSI, DWSI, DWDI, panoramic, projection, stereo),

and any special considerations.

E) GENERATE the 3D_render_hint for each primitive:

This tells the renderer which Three.js geometry to create.

Use ONLY these constructors:

CylinderGeometry(radiusTop, radiusBot, height, segments)

SphereGeometry(radius, wSeg, hSeg, phiStart, phiLen,

thetaStart, thetaLen)

TorusGeometry(radius, tube, radialSeg, tubularSeg, arc)

BoxGeometry(width, height, depth)

ConeGeometry(radius, height, segments)

LatheGeometry(points_array, segments) - for custom profiles

ExtrudeGeometry(shape_points, depth) - for beam cross-sections

OUTPUT FORMAT:

Respond with ONLY valid JSON matching the schema in the user

message. No markdown fences, no commentary.

## 4.2 Code-Specific Modules (Selected by Stage 1)

Each module is a text file loaded from disk and appended to the base system prompt. They live at /prompts/modules/{module_name}.txt in the codebase. Here are the key modules:

### 4.2.1 Module: asme_viii_vessel

CODE-SPECIFIC MODULE: ASME BPVC Section VIII Division 1 & 2

WELD JOINT CATEGORIES per UW-3:

Category A: Longitudinal/meridional joints in shells, heads,

transition sections, nozzles, communicating chambers

Category B: Circumferential joints in shells, heads, transitions,

nozzles, communicating chambers

Category C: Flange-to-shell/head/nozzle joints

Category D: Nozzle-to-shell/head attachment welds

RT EXTENT per UW-11:

UW-11(a)(1): Full RT - all butt welds, E = 1.0

UW-11(a)(2): Full RT Cat A & D only, spot Cat B, E = 1.0/0.85

UW-11(a)(5): Spot RT, E = 0.85

UW-11(b): No RT required, E = 0.70

ACCEPTANCE CRITERIA: UW-51(b), Appendix 4

IQI REQUIREMENTS: T-233.2 (source-side), T-233.3 (film-side)

GEOMETRIC UNSHARPNESS: T-274.1, Ug <= 1/16" (< 2" thk)

ALWAYS CHECK:

- UCS-56 PWHT requirements based on P-No, thickness, preheat

- UCS-66 MDMT / impact test requirements

- UCS-68 carbon content limitations

- UW-40 special requirements for unfired steam boilers

- UG-20 lethal service designations

### 4.2.2 Module: asme_b31_piping

CODE-SPECIFIC MODULE: ASME B31.3 Process Piping

EXAMINATION EXTENT per Table 341.3.2:

Normal service: 5% random RT of butt welds

Severe cyclic: 100% RT of all butt welds

Category M (lethal): 100% RT of all butt welds

High pressure: per Chapter IX requirements

WELD TYPES:

Butt welds (girth and longitudinal)

Branch connection welds (set-in, set-on, integrally reinforced)

Socket welds (not typically RT - note limitation)

Fillet welds (not typically RT - note limitation)

ACCEPTANCE CRITERIA: Table 341.3.2 + ASME V Article 2

SPECIAL: In-service inspection per B31.3 Chapter VII if applicable

GEOMETRY NOTES:

- Elbows: use torus_segment primitive with bend radius and angle

- Reducers: use cone primitive or compound cone+cylinder

- Tees: compound cylinder intersection

- Flanges: disk primitive with bolt_hole_pattern features

### 4.2.3 Module: aws_structural

CODE-SPECIFIC MODULE: AWS D1.1 Structural Welding

WELD CLASSIFICATIONS:

CJP (Complete Joint Penetration): Full RT per Clause 6

PJP (Partial Joint Penetration): Generally not RT inspected

Fillet welds: Not RT inspected (use UT/MT/PT)

RT REQUIREMENTS per Clause 6.12:

Statically loaded: RT per engineer's discretion

Cyclically loaded (fatigue): RT per Table 6.1

Fracture critical: 100% RT of all CJP welds

ACCEPTANCE CRITERIA: Clause 6.12.2 + Table 6.2

DISCONTINUITY CATEGORIES: elongated, rounded, cluster

GEOMETRY NOTES:

- Use plate + beam primitives for I-beams, channels, angles

- Use ExtrudeGeometry for built-up sections

- Gussets and stiffeners are plate primitives

- Connection zones (beam-to-column) are highest priority

### 4.2.4 Module: casting_radiography

CODE-SPECIFIC MODULE: Casting Radiography

REFERENCE RADIOGRAPHS:

ASTM E446: Steel castings up to 2" (51mm)

ASTM E186: Steel castings 2" to 4.5" (51-114mm)

ASTM E280: Steel castings 4.5" to 12" (114-305mm)

ASTM E2422: Digital reference images for castings

DISCONTINUITY TYPES (per category):

Category A: Gas porosity (Types A1-A5 severity)

Category B: Sand/slag inclusions (B1-B5)

Category C: Shrinkage (C1-C5)

Category D: Shrinkage + gas (D1-D5)

Category E: Cracks (accept/reject only)

Category F: Inserts/chaplets

Category G: Mottling

CRITICAL ZONES IN CASTINGS:

- Section thickness transitions (hot spots)

- Last-to-solidify regions (centerline shrinkage)

- Gate and riser removal areas

- Core support / chaplet locations

- Sharp internal corners (stress risers + shrinkage)

GEOMETRY NOTES:

- Use freeform_solid primitive - provide bounding box

- Identify internal passages as subtracted volumes

- Section thickness map is critical for technique planning

- RT may need multiple angles for complex geometry

### 4.2.5 Module: aerospace_ndt

CODE-SPECIFIC MODULE: Aerospace NDT

GOVERNING STANDARDS:

NAS 410 / EN 4179: Personnel qualification

Nadcap AC7114: RT process audit checklist

AMS 2175: Casting inspection

AMS 2635: RT of turbine components

Customer prime specs (Boeing BAC, Airbus AIMS, etc.)

TECHNIQUES:

Conventional film RT: MIL-STD-453 heritage

Digital radiography (DR): ASTM E2597

Computed radiography (CR): ASTM E2007

Computed tomography (CT): ASTM E1695

CRITICAL FOR AEROSPACE:

- FOD detection (foreign object debris in assemblies)

- Braze joint coverage verification

- Composite delamination / disbond (X-ray limited)

- Tight tolerance defect sizing for fracture mechanics

- Part-specific written procedures (not generic)

GEOMETRY NOTES:

- Airfoils: use LatheGeometry or custom profile

- Skin panels: flat plate primitives

- Spars/ribs: extruded beam sections

- Engine components: disk, ring, shaft primitives

- Complex assemblies may need CT, not projection RT

# 5. STAGE 2 UNIVERSAL OUTPUT SCHEMA

This is the JSON schema that Stage 2 must produce regardless of part type. It replaces the vessel-specific schema from v1.0 with a geometry-agnostic structure that the renderer’s geometry factory can consume.

{

"part_id": "string",

"part_type": "string",

"analysis_module_used": "string",

"analysis_summary": "string (2-3 sentences)",

"render_model": {

"primitives": [

{

"id": "string (matches Stage 1 primitive id)",

"three_js": {

"geometry": "CylinderGeometry|SphereGeometry|...",

"params": [0, 0, 0],

"position": [0, 0, 0],

"rotation": [0, 0, 0],

"scale": [1, 1, 1]

},

"material_appearance": {

"color": "#hex",

"opacity": 0.0-1.0,

"metalness": 0.0-1.0

}

}

]

},

"inspection_zones": [

{

"id": "string",

"type": "string (butt_weld|fillet_weld|casting_zone|...)",

"severity": "CRITICAL|HIGH|MEDIUM|LOW",

"on_primitive": "string (primitive id this zone sits on)",

"geometry_type": "ring|line|patch|sphere|arc",

"position": {

"x_normalized": 0.0,

"y_normalized": 0.0,

"z_normalized": 0.0,

"angle_degrees": 0,

"span_degrees": 360

},

"code_classification": {

"category": "string (e.g. A, B, C, D, CJP, PJP, N/A)",

"rt_requirement": "100%|SPOT|RANDOM_5%|NONE|CT",

"joint_efficiency": 0.0-1.0,

"acceptance_standard": "string (e.g. UW-51(b))"

},

"expected_defects": [

{

"type": "string",

"probability": "HIGH|MEDIUM|LOW",

"description": "string",

"code_reference": "string",

"severity_grade": "string (for castings: A1-A5, etc.)"

}

],

"rt_technique": {

"source": "Ir-192|Co-60|X-ray|micro-focus|linear_accel",

"energy_range": "string (e.g. 160-200 kV)",

"technique": "SWSI|DWSI|DWDI|panoramic|CT|DR|CR",

"detector": "film|CR_plate|DDA_panel|CT_detector",

"iqi_type": "wire|hole|duplex_wire|step_wedge",

"iqi_placement": "source_side|film_side",

"sfd_inches": 0,

"geometric_unsharpness_limit": "string",

"notes": "string"

},

"overlay_render": {

"color": "#hex (based on severity)",

"opacity": 0.0-1.0,

"pulse_animation": true,

"line_width": 0

},

"tooltip_text": "string (concise field-ready note)"

}

],

"critical_intersections": [

{

"id": "string",

"zone_a": "string (inspection_zone id)",

"zone_b": "string (inspection_zone id)",

"severity": "CRITICAL",

"position": {

"x_normalized": 0.0,

"angle_degrees": 0

},

"defect_risks": ["string"],

"shot_technique": "string",

"tooltip_text": "string"

}

],

"compliance_flags": [

{

"code_paragraph": "string",

"condition": "string",

"action_required": "string",

"flag_type": "mandatory|advisory|informational"

}

],

"shot_plan": {

"total_exposures": 0,

"source_recommendation": "string",

"estimated_film_count": 0,

"coverage_strategy": "string",

"special_techniques": ["string"]

}

}

# 6. GEOMETRY FACTORY (RENDERER)

The v1.0 renderer hardcoded a cylinder + two elliptical heads. The v2.0 renderer uses a geometry factory that reads the render_model.primitives array from Stage 2 output and constructs any part from the supported Three.js primitive set.

## 6.1 Factory Pattern (TypeScript)

// geometry-factory.ts

import * as THREE from "three";

type PrimitiveSpec = {

id: string;

three_js: {

geometry: string;

params: number[];

position: [number, number, number];

rotation: [number, number, number];

scale: [number, number, number];

};

material_appearance: {

color: string;

opacity: number;

metalness: number;

};

};

const GEOMETRY_MAP: Record<string,

(...args: number[]) => THREE.BufferGeometry> = {

CylinderGeometry: (rT, rB, h, s) =>

new THREE.CylinderGeometry(rT, rB, h, s || 64),

SphereGeometry: (r, ws, hs, ps, pl, ts, tl) =>

new THREE.SphereGeometry(r, ws||64, hs||32,

ps||0, pl||Math.PI*2, ts||0, tl||Math.PI),

TorusGeometry: (r, t, rs, ts, a) =>

new THREE.TorusGeometry(r, t, rs||16, ts||64,

a||Math.PI*2),

BoxGeometry: (w, h, d) =>

new THREE.BoxGeometry(w, h, d),

ConeGeometry: (r, h, s) =>

new THREE.ConeGeometry(r, h, s || 64),

};

export function buildPartFromSpec(

primitives: PrimitiveSpec[]

): THREE.Group {

const group = new THREE.Group();

for (const prim of primitives) {

const factory = GEOMETRY_MAP[prim.three_js.geometry];

if (!factory) {

console.warn(`Unknown geometry: ${prim.three_js.geometry}`);

continue;

}

const geo = factory(...prim.three_js.params);

const mat = new THREE.MeshStandardMaterial({

color: prim.material_appearance.color,

transparent: true,

opacity: prim.material_appearance.opacity,

metalness: prim.material_appearance.metalness,

roughness: 1 - prim.material_appearance.metalness,

side: THREE.DoubleSide,

});

const mesh = new THREE.Mesh(geo, mat);

mesh.position.fromArray(prim.three_js.position);

mesh.rotation.fromArray(

prim.three_js.rotation.map(d => d * Math.PI / 180)

);

mesh.scale.fromArray(prim.three_js.scale);

mesh.userData.primitiveId = prim.id;

group.add(mesh);

}

return group;

}

## 6.2 Overlay Factory

// overlay-factory.ts

const OVERLAY_GEOMETRY: Record<string,

(zone, prim) => THREE.Mesh> = {

ring: (zone, prim) => {

// Circumferential weld on a cylinder

const R = prim.three_js.params[0]; // radiusTop

const geo = new THREE.TorusGeometry(R + 0.1, 0.1, 8, 128);

geo.rotateY(Math.PI / 2);

const mesh = new THREE.Mesh(geo, overlayMat(zone));

mesh.position.x = mapNormToWorld(

zone.position.x_normalized, prim);

return mesh;

},

line: (zone, prim) => {

// Longitudinal weld along a cylinder

const R = prim.three_js.params[0];

const L = prim.three_js.params[2];

const angle = zone.position.angle_degrees * Math.PI / 180;

const points = [];

for (let t = -L/2; t <= L/2; t += 0.5) {

points.push(new THREE.Vector3(

t, Math.cos(angle)*(R+0.1), Math.sin(angle)*(R+0.1)));

}

const curve = new THREE.CatmullRomCurve3(points);

const geo = new THREE.TubeGeometry(curve, 64, 0.12, 8);

return new THREE.Mesh(geo, overlayMat(zone));

},

patch: (zone, prim) => {

// Area zone on a surface (casting hot spot, head crown)

const geo = new THREE.SphereGeometry(1.5, 32, 16);

const mesh = new THREE.Mesh(geo, overlayMat(zone));

mesh.position.set(

mapNormToWorld(zone.position.x_normalized, prim),

mapNormToWorld(zone.position.y_normalized, prim),

mapNormToWorld(zone.position.z_normalized, prim)

);

return mesh;

},

sphere: (zone, prim) => {

// Point feature (intersection, defect location)

const geo = new THREE.SphereGeometry(0.35, 16, 16);

return new THREE.Mesh(geo, overlayMat(zone));

},

arc: (zone, prim) => {

// Partial circumferential feature

const R = prim.three_js.params[0];

const span = zone.position.span_degrees * Math.PI / 180;

const start = zone.position.angle_degrees * Math.PI / 180;

const geo = new THREE.TorusGeometry(

R+0.1, 0.1, 8, 64, span);

geo.rotateY(Math.PI / 2);

geo.rotateX(start);

return new THREE.Mesh(geo, overlayMat(zone));

},

};

# 7. TEMPORAL WORKFLOW (UPDATED FOR TWO-STAGE)

// workflow: universalRTAnalysis

async function universalRTAnalysis(drawingPath: string) {

// Step 1: OCR + Layout Extraction

const rawData = await activities.extractDrawingData(drawingPath);

// Step 2: ITAR Compliance Check

const compliance = await activities.checkITARCompliance(rawData);

const useLocal = compliance.isITAR;

// Step 3: Sanitize if going to cloud

const cleanData = useLocal

? rawData

: await activities.sanitizeForCloud(rawData);

// ===== STAGE 1: CLASSIFICATION =====

const classification = await activities.classifyPart({

drawingData: cleanData,

useLocal,

model: useLocal ? "ndt-classifier" : "claude-sonnet-4-20250514",

maxRetries: 2,

});

// Validate classification

await activities.validateClassification(classification);

// ===== PROMPT ASSEMBLY =====

const moduleFile = `./prompts/modules/${classification.analysis_module}.txt`;

const systemPrompt = await activities.assembleSystemPrompt({

basePrompt: "./prompts/base-rt-analyst.txt",

moduleFile,

});

// ===== STAGE 2: RT ANALYSIS =====

const analysis = await activities.runRTAnalysis({

classification,

systemPrompt,

useLocal,

model: useLocal ? "ndt-rt-analyst" : "claude-sonnet-4-20250514",

fallbackModel: "claude-opus-4-6",

maxRetries: 2,

});

// Validate analysis output

await activities.validateAnalysis(analysis, classification);

// ===== RENDER + REPORT =====

const overlayConfig = await activities.buildOverlayConfig(analysis);

const report = await activities.generateReport(

analysis, classification);

// Cache for fast retrieval

await activities.cacheResult({

partId: classification.part_id,

classification,

analysis,

overlayConfig,

reportPath: report.path,

});

return { classification, analysis, overlayConfig, reportPath: report.path };

}

# 8. PROMPT ASSEMBLY LOGIC

This is the critical glue between Stage 1 and Stage 2. The assembler reads the analysis_module from Stage 1 output, loads the matching module file, and concatenates it with the base system prompt.

## 8.1 Assembler Code

// prompt-assembler.ts

import fs from "fs/promises";

import path from "path";

const PROMPTS_DIR = "./prompts";

const MODULES_DIR = path.join(PROMPTS_DIR, "modules");

const MODULE_MAP: Record<string, string> = {

asme_viii_vessel:    "asme_viii_vessel.txt",

asme_b31_piping:    "asme_b31_piping.txt",

aws_structural:     "aws_structural.txt",

casting_radiography: "casting_radiography.txt",

forging_inspection:  "forging_inspection.txt",

aerospace_ndt:       "aerospace_ndt.txt",

heat_exchanger:      "heat_exchanger.txt",

api_tank:            "api_tank.txt",

generic_rt:          "generic_rt.txt",

};

export async function assembleSystemPrompt(

analysisModule: string

): Promise<string> {

const base = await fs.readFile(

path.join(PROMPTS_DIR, "base-rt-analyst.txt"), "utf-8");

const moduleFile = MODULE_MAP[analysisModule];

if (!moduleFile) {

throw new Error(`Unknown module: ${analysisModule}`);

}

const module = await fs.readFile(

path.join(MODULES_DIR, moduleFile), "utf-8");

return `${base}\n\n---\n\n${module}`;

}

// Stage 2 user prompt wraps the Stage 1 output

export function buildStage2UserPrompt(

classification: PartClassification

): string {

return `

Analyze this classified part for RT inspection.

<part_classification>

${JSON.stringify(classification, null, 2)}

</part_classification>

Generate the complete RT analysis with 3D render hints.

Respond with ONLY valid JSON matching the universal schema.

`.trim();

}

## 8.2 File Structure

prompts/

├── base-rt-analyst.txt          # Section 4.1 base prompt

├── stage1-classifier.txt        # Section 3.1 classification prompt

├── stage1-user-template.txt     # Section 3.2 user template

└── modules/

├── asme_viii_vessel.txt      # Pressure vessels

├── asme_b31_piping.txt      # Process piping

├── aws_structural.txt       # Structural steel

├── casting_radiography.txt  # Castings

├── forging_inspection.txt   # Forgings

├── aerospace_ndt.txt        # Aerospace components

├── heat_exchanger.txt       # TEMA heat exchangers

├── api_tank.txt             # Storage tanks

└── generic_rt.txt           # Catch-all fallback

# 9. VALIDATION & FAILURE MODES (UPDATED)

## 9.1 Stage 1 Validation

| Check | Rule | On Failure |
| --- | --- | --- |
| part_type valid | Must be one of the 9 allowed values | Retry with explicit list of options |
| primitives non-empty | At least 1 primitive in geometry | Retry. If still empty, flag for human review. |
| features non-empty | At least 1 inspectable feature | Accept — some forgings/castings have no welds |
| analysis_module valid | Must map to a module file that exists on disk | Fall back to generic_rt module |
| confidence threshold | confidence >= 0.6 | If < 0.6, present classification to user for confirmation before Stage 2 |
| dimension sanity | No dimension is 0 or negative, units consistent | Re-extract from OCR with focused prompt |

## 9.2 Stage 2 Validation

| Check | Rule | On Failure |
| --- | --- | --- |
| JSON schema | Validates against universal schema (Zod/ajv) | Retry up to 2x, then fallback to Opus |
| Primitive IDs match | Every on_primitive in inspection_zones must reference a primitive from render_model | Strip orphaned zones, log warning |
| Position bounds | All normalized values 0.0–1.0, angles 0–360 | Clamp to bounds, log warning |
| Severity monotonicity | Intersections >= max(zone_a, zone_b) severity | Auto-elevate in post-processing |
| Code ref validity | Cross-check code_reference against known paragraph lists per module | Strip invalid refs, keep zone |
| Three.js params valid | Geometry constructor params are positive numbers, correct arity | Fall back to bounding-box BoxGeometry |
| Overlay color match | overlay_render.color matches severity mapping | Override with canonical severity colors |
| RT technique plausible | Source energy appropriate for material thickness | Log advisory flag, keep analysis |

## 9.3 New Failure Modes (v2.0 Additions)

| Failure Mode | Detection | Mitigation |
| --- | --- | --- |
| Wrong part classification | confidence < 0.6 or user override | Present classification to user before Stage 2. Allow manual correction. |
| Unknown geometry type | Part doesn’t fit any primitive set | Use freeform_solid with bounding box. Flag for manual 3D model import (STEP/STL). |
| Code not in module set | analysis_module = “other” or unknown code referenced | Use generic_rt module. Log for module development backlog. |
| Multi-code part | Stage 1 returns multiple primary codes | Use most restrictive code as primary. Include others in supplementary. |
| Stage 1/2 disagreement | Stage 2 geometry doesn’t match Stage 1 primitives | Stage 1 primitives are authoritative for render. Stage 2 zones must reference them. |
| 3D render params cause crash | Three.js throws on construction | Wrap factory in try/catch. Fall back to wireframe bounding box for failed primitives. |

# 10. MODEL SELECTION (UPDATED)

| Stage | Model | Reason | Cost/Call |
| --- | --- | --- | --- |
| Stage 1 (Cloud) | Sonnet 4 | Classification is structured extraction. Sonnet handles it efficiently. | ~$0.004 |
| Stage 2 (Cloud) | Sonnet 4 | Analysis with module context. Best cost/quality. | ~$0.012 |
| Stage 2 Fallback | Opus 4.6 | Complex geometry or failed retries. | ~$0.08 |
| Stage 1 (ITAR) | Qwen3-32B / Ollama | On-prem. Create ndt-classifier Modelfile. | $0 (GPU time) |
| Stage 2 (ITAR) | Qwen3-32B / Ollama | On-prem. Create ndt-rt-analyst Modelfile. | $0 (GPU time) |
| Batch (overnight) | Sonnet 4 Batch API | 50% cost reduction for bulk processing. | ~$0.008 total |

Total cost per part analysis (cloud): approximately $0.016 for both stages at Sonnet tier. At 100 parts/month = ~$1.60/month in LLM costs.

# 11. CLAUDE CODE DIRECTIVE

Drop this into your CLAUDE.md for the project:

# NDT RT SCAN ANALYZER

## Architecture

Two-stage LLM pipeline for universal part RT analysis.

Stage 1: Part classification + geometry extraction.

Stage 2: RT analysis with code-specific prompt modules.

## Stack

- Frontend: Next.js 14 App Router + @react-three/fiber +

@react-three/drei + shadcn/ui + Tailwind

- Analysis API: FastAPI (Python) — handles OCR, Presidio,

Ollama routing, prompt assembly

- Orchestration: Temporal (TypeScript SDK)

- Data: PostgreSQL + pgvector + Hasura GraphQL

- LLM: Anthropic SDK (Sonnet 4 primary, Opus 4.6 fallback)

+ Ollama (Qwen3-32B for ITAR)

- OCR: Surya + PaddleOCR

- Reports: docx-js + Puppeteer

## DO NOT

- Hardcode geometry to any specific part type

- Use raw Three.js DOM — always use R3F declarative

- Use n8n for this workflow — Temporal only

- Send ITAR-flagged data to cloud APIs

- Use temperature > 0.3 for any LLM call

- Trust LLM code references without validation

## File Structure

prompts/                    # All LLM prompts

base-rt-analyst.txt       # Stage 2 base system prompt

stage1-classifier.txt     # Stage 1 system prompt

modules/                  # Code-specific modules

src/

lib/prompt-assembler.ts   # Concatenates base + module

lib/geometry-factory.ts   # Three.js from LLM JSON

lib/overlay-factory.ts    # Inspection zones to meshes

lib/validator.ts          # Zod schemas for both stages

workflows/                # Temporal workflow definitions

components/               # R3F components

VesselViewer.tsx         # (rename to PartViewer.tsx)

OverlayLayer.tsx

InspectionTooltip.tsx

ControlPanel.tsx

END OF BUILD SHEET v2.0

Supersedes v1.0. All vessel-specific assumptions removed. Test Stage 1 classification accuracy against 10+ diverse part drawings (vessel, pipe spool, casting, structural) before deploying Stage 2.