RT MACHINE TECHNICAL SPECIFICATIONS

Research & Configuration Report

Prepared by: Onnex AI Agency  ·  NDT Portal v1 Project  ·  April 01, 2026

Executive Summary

This report documents the technical specifications for four (4) industrial radiographic testing (RT) X-ray machines operated at the client's facility. Specifications were gathered from two sources:

1.  Customer-provided data — submitted via machine specification request form.

2.  Manufacturer datasheet research — supplemented from official Varex Imaging and Comet Group product documentation, where customer responses did not include tube-level parameters required for automated technique planning.

Fields supplemented from external research are marked with an asterisk (*) throughout this document. All supplemented values are sourced from publicly available manufacturer datasheets and are referenced in the Sources section of each machine profile.

These specifications have been loaded into the NDT Portal v1 machine catalog and are used by the AI-assisted RT technique planner to select the appropriate machine, calculate geometric unsharpness (Ug), and generate technique cards per ASME Section V / ASTM E1742 requirements.

Machine Profiles

Unit 1 — Varian NDI 320/26  (Walk-In Vault)

Machine ID: RT_VARIAN_320_26  ·  Make/Model: Varian / NDI 320/26  ·  Chamber Type: Walk-In Vault

★  Fields marked * were supplemented from the Varex NDI-320-26 Product Data Sheet (vareximaging.com) and xrayllc.com distributor datasheet, as the customer's specification response did not include these tube-level parameters.

1. X-Ray Source

| Type | NDT — Industrial X-ray |
| --- | --- |
| Maximum voltage | 320 kV |
| Operating range | 40 – 300 kV (customer-provided) |
| Max mA at rated voltage | 10 mA (customer-provided) |
| Focal spot class | Standard industrial |
| Focal spot size (small) * * | 1.5 mm (IEC 336 / EN 12543) |
| Focal spot size (large) * * | 4.0 mm (IEC 336 / EN 12543) |
| Beam cone angle * * | 40 degrees |
| Target / anode angle * * | 20 degrees |
| Target material * * | Tungsten — stationary anode, metal-ceramic |
| Inherent filtration * * | 4.0 mm Be (beryllium window) |
| Max continuous power (small focal) * * | 1,500 W (@ 14 L/min oil flow) |
| Max continuous power (large focal) * * | 4,200 W (@ 14 L/min oil flow) |
| Cooling * * | Oil cooled — 14 L/min minimum flow |
| Modality | Film RT (film_rt) |

2. Inspection Envelope

| Chamber shape | Rectangular — Walk-In Vault |
| --- | --- |
| Max part diameter | 914.4 mm (36 inches) |
| Max part height | 1,526 mm (60 inches) |
| Max part weight | 22.7 kg (50 lbs) |
| Usable clearance | 36 inches — part + fixturing |
| FFD (Focus-to-Film Distance) | 64 inches (machine log reference) |

3. Manipulation

| Available axes | Rotate, Vertical, Horizontal, Tilt |
| --- | --- |
| Tilt available | Yes |
| Rotation range | 0 – 90 degrees |

4. Detector Support

| Film supported | Yes |
| --- | --- |
| Digital detector | No |
| Typical film classes | DR, D2, D3, D4, D5, D7, IX50, D4 ETE PB |
| IQI support | IQI Wire, IQI Hole |

5. Planning Rules & Machine Suitability

| Best for | Large parts, long pipe spools, parts requiring tilt, multi-axis manipulation |
| --- | --- |
| Not ideal for | Gamma source jobs (Ir-192 / Co-60 not applicable to this cabinet) |

Sources

Varex NDI-320-26 Product Data Sheet (official)  https://www.vareximaging.com/wp-content/uploads/2022/01/NDI-320-26_PDS_133112-000.pdf

xrayllc.com — NDI 320/26 Distributor Datasheet  https://xrayllc.com/ndi320-26.pdf

Unit 2 — Comet MXR225/22  (Cabinet)

Machine ID: RT_COMET_MXR225_22  ·  Make/Model: Comet MXR225/22  ·  Chamber Type: Cabinet

★  Fields marked * were supplemented from the Comet Group official product page (xray.comet.tech) and a verified distributor technical listing.

1. X-Ray Source

| Type | NDT — Industrial X-ray |
| --- | --- |
| Maximum voltage | 225 kV |
| Operating range | 40 – 220 kV (customer-provided) |
| Max mA at rated voltage | 10 mA (customer-provided) |
| Focal spot class | Standard industrial |
| Focal spot size (small) * * | 1.0 mm (EN 12543) |
| Focal spot size (large) * * | 5.5 mm (EN 12543) |
| Beam cone angle * * | 40 degrees |
| Target / anode angle * * | 20 degrees |
| Target material * * | Tungsten — stationary anode, metal-ceramic, unipolar |
| Inherent filtration * * | 0.8 ± 0.1 mm Be (beryllium window) |
| Max continuous power (small focal) * * | 640 W |
| Max continuous power (large focal) * * | 3,000 W |
| Cooling * * | Water cooled — 4 L/min minimum flow |
| Modality | Film RT (film_rt) |

2. Inspection Envelope

| Chamber shape | Rectangular — Cabinet |
| --- | --- |
| Max part diameter | 304.8 mm (12 inches) |
| Max part height | 304.8 mm (12 inches) |
| Max part weight | 22.68 kg |
| Usable clearance | 12 inches |
| FFD (Focus-to-Film Distance) | 44 inches (machine log reference) |

3. Manipulation

| Available axes | Vertical only |
| --- | --- |
| Tilt available | No |
| Rotation | N/A |

4. Detector Support

| Film supported | Yes |
| --- | --- |
| Digital detector | No |
| Typical film classes | DR, D2, D3, D4, D5, D7, IX50, D4 ETE PB |
| IQI support | IQI Wire, IQI Hole |

5. Planning Rules & Machine Suitability

| Best for | Small parts, compact geometry, lower-energy jobs |
| --- | --- |
| Not ideal for | Gamma source jobs; parts exceeding 304.8 mm envelope |

Sources

Comet MXR-225/22 Official Product Page  https://xray.comet.tech/en/products/mxr-225-22

Comet MXR-225/22 Technical Datasheet — Distributor (made-in-china.com)  https://zxt-xray.en.made-in-china.com/product/CdIAbmflHxco/China-Comet-X-ray-Tube-for-X-ray-Machine-225kv-Mxr-225-22-.html

Unit 3 — Comet MXR320/26  (Cabinet)

Machine ID: RT_COMET_MXR320_26  ·  Make/Model: Comet MXR320/26  ·  Chamber Type: Cabinet

★  Fields marked * were supplemented from the Comet Group official product page (xray.comet.tech) and the Willick Engineering distributor datasheet.

1. X-Ray Source

| Type | NDT — Industrial X-ray |
| --- | --- |
| Maximum voltage | 320 kV |
| Operating range | 40 – 300 kV (customer-provided) |
| Max mA at rated voltage | 10 mA (customer-provided) |
| Focal spot class | Standard industrial |
| Focal spot size (small) * * | 3.0 mm (EN 12543) |
| Focal spot size (large) * * | 5.5 mm (EN 12543) |
| Beam cone angle * * | 40 degrees |
| Target / anode angle * * | 20 degrees |
| Target material * * | Tungsten — stationary anode, metal-ceramic, bipolar |
| Inherent filtration * * | 3.0 mm Be (beryllium window) |
| Max continuous power (small focal) * * | 1,500 W |
| Max continuous power (large focal) * * | 4,200 W |
| Cooling * * | Oil cooled — 14 L/min minimum, max inlet temp 50 °C |
| Modality | Film RT (film_rt) |

2. Inspection Envelope

| Chamber shape | Rectangular — Cabinet |
| --- | --- |
| Max part diameter | 457.2 mm (18 inches) |
| Max part height | 304.8 mm (12 inches) |
| Max part weight | 22.68 kg |
| Usable clearance | 12 inches |
| FFD (Focus-to-Film Distance) | 44 inches (machine log reference) |

3. Manipulation

| Available axes | Vertical only |
| --- | --- |
| Tilt available | No |
| Rotation | N/A |

4. Detector Support

| Film supported | Yes |
| --- | --- |
| Digital detector | No |
| Typical film classes | DR, D2, D3, D4, D5, D7, IX50, D4 ETE PB |
| IQI support | IQI Wire, IQI Hole |

5. Planning Rules & Machine Suitability

| Best for | Medium-diameter parts, thick sections requiring 320 kV penetration |
| --- | --- |
| Not ideal for | Gamma source jobs; parts taller than 304.8 mm |

Sources

Comet MXR-320/26 Official Product Page  https://xray.comet.tech/en/products/mxr-320-26

Comet MXR-320/26 Datasheet via Willick Engineering  https://www.willick.com/wp-content/uploads/2021/05/comet-mxr-320_26_single_sheet_en_v6.pdf

Unit 4 — Varex NDI 320/26  (Cabinet)

Machine ID: RT_VAREX_NDI320_26  ·  Make/Model: Varex NDI 320/26  ·  Chamber Type: Cabinet

★  Fields marked * were supplemented from the Varex NDI-320-26 Product Data Sheet. Note: Varex Imaging acquired Varian Medical Systems' Imaging Components division in 2017. The NDI 320/26 is the same X-ray tube product now manufactured and sold under the Varex brand. All tube-level specifications are identical to Unit 1.

1. X-Ray Source

| Type | NDT — Industrial X-ray |
| --- | --- |
| Maximum voltage | 320 kV |
| Operating range | 40 – 300 kV (customer-provided) |
| Max mA at rated voltage | 10 mA (customer-provided) |
| Focal spot class | Standard industrial |
| Focal spot size (small) * * | 1.5 mm (IEC 336 / EN 12543) |
| Focal spot size (large) * * | 4.0 mm (IEC 336 / EN 12543) |
| Beam cone angle * * | 40 degrees |
| Target / anode angle * * | 20 degrees |
| Target material * * | Tungsten — stationary anode, metal-ceramic |
| Inherent filtration * * | 4.0 mm Be (beryllium window) |
| Max continuous power (small focal) * * | 1,500 W (@ 14 L/min oil flow) |
| Max continuous power (large focal) * * | 4,200 W (@ 14 L/min oil flow) |
| Cooling * * | Oil cooled — 14 L/min minimum flow |
| Modality | Film RT (film_rt) |

2. Inspection Envelope

| Chamber shape | Rectangular — Cabinet |
| --- | --- |
| Max part diameter | 457.2 mm (18 inches) |
| Max part height | 304.8 mm (12 inches) |
| Max part weight | 22.68 kg |
| Usable clearance | 12 inches |
| FFD (Focus-to-Film Distance) | 48 inches (machine log reference) |

3. Manipulation

| Available axes | Vertical only |
| --- | --- |
| Tilt available | No |
| Rotation | N/A |

4. Detector Support

| Film supported | Yes |
| --- | --- |
| Digital detector | No |
| Typical film classes | DR, D2, D3, D4, D5, D7, IX50, D4 ETE PB |
| IQI support | IQI Wire, IQI Hole |

5. Planning Rules & Machine Suitability

| Best for | Medium-diameter parts, thick sections requiring 320 kV penetration |
| --- | --- |
| Not ideal for | Gamma source jobs; parts taller than 304.8 mm |

Sources

Varex NDI-320-26 Product Data Sheet (official)  https://www.vareximaging.com/wp-content/uploads/2022/01/NDI-320-26_PDS_133112-000.pdf

Varex Imaging Corporate — former Varian NDT division  https://www.vareximaging.com

Comparative Analysis

The table below summarises the key differentiating parameters across all four machines. Parameters marked * are supplemented from manufacturer datasheets.

| Parameter | Unit 1 Varian NDI 320/26 (Walk-In) | Unit 2 Comet MXR225/22 (Cabinet) | Unit 3 Comet MXR320/26 (Cabinet) | Unit 4 Varex NDI 320/26 (Cabinet) |
| --- | --- | --- | --- | --- |
| Max kV | 320 kV | 225 kV | 320 kV | 320 kV |
| Max mA | 10 mA | 10 mA | 10 mA | 10 mA |
| FFD (machine log) | 64 inches | 44 inches | 44 inches | 48 inches |
| Chamber type | Walk-In | Cabinet | Cabinet | Cabinet |
| Max part diameter | 914 mm | 305 mm | 457 mm | 457 mm |
| Max part height | 1,526 mm | 305 mm | 305 mm | 305 mm |
| Axes | 4 (full) | Vertical | Vertical | Vertical |
| Tilt | Yes | No | No | No |
| Focal spot — small * | 1.5 mm | 1.0 mm | 3.0 mm | 1.5 mm |
| Focal spot — large * | 4.0 mm | 5.5 mm | 5.5 mm | 4.0 mm |
| Beam cone angle * | 40° | 40° | 40° | 40° |
| Target angle * | 20° | 20° | 20° | 20° |
| Inherent filtration * | 4.0 mm Be | 0.8 mm Be | 3.0 mm Be | 4.0 mm Be |
| Power — small focal * | 1,500 W | 640 W | 1,500 W | 1,500 W |
| Power — large focal * | 4,200 W | 3,000 W | 4,200 W | 4,200 W |
| Cooling * | Oil | Water | Oil | Oil |
| Film only | Yes | Yes | Yes | Yes |

Technical Planning Implications

Geometric Unsharpness (Ug)

Ug is calculated as:  Ug = F × (OFD / SFD)  where F = focal spot size in mm, OFD = object-to-film distance, SFD = source-to-film distance.

The Comet MXR225/22 offers the smallest focal spot (1.0 mm small / 5.5 mm large). At small focal spot with equal geometry, it achieves the lowest Ug of all four machines — best for fine defect detection in thin-walled parts. The Comet MXR320/26 has the largest small focal spot (3.0 mm), producing the highest Ug at equivalent geometry — suitable for thick sections where penetration is the priority.

Beam Coverage at Given FFD

All four tubes share a 40° cone angle. Coverage diameter = 2 × SFD × tan(20°). At the machine-logged FFDs:

Unit 1 (Walk-In) — FFD 64":  coverage diameter ≈ 46.6 inches

Unit 4 (Varex Cabinet) — FFD 48":  coverage diameter ≈ 34.9 inches

Units 2 & 3 (Comet Cabinets) — FFD 44":  coverage diameter ≈ 32.0 inches

Beam Hardening & Filtration

Inherent filtration varies significantly across machines. The Comet MXR225/22 has only 0.8 mm Be filtration, producing a softer beam. For thick steel sections this may require additional external filtration (e.g. Cu or Al filters) to reduce scatter and beam hardening artefacts. The Varian/Varex NDI 320/26 tubes (4.0 mm Be) produce the most pre-hardened beam — less scatter, better contrast in dense materials.

Power-Limited Exposure at Small Focal Spot

At small focal spot, maximum continuous power limits effective mA at high kV. At 225 kV, the Comet MXR225/22 is limited to 640 W → max ~2.8 mA. At 320 kV, Varian/Varex and Comet MXR320/26 reach 1,500 W → max ~4.7 mA. Longer exposure times are required at small focal spot — a factor in technique card generation.

Known Data Gaps

The following parameters were not available in public manufacturer documentation at time of research. Obtaining these from the client or manufacturer would improve technique planning accuracy:

Radiation output curves (mGy/min or R/min at 1 m at rated kV/mA) — required for exposure time calculation

mA minimum range and step resolution — generator specifications, not tube specs

Duty cycle / maximum single-exposure time — operating manuals required

Physical cabinet dimensions (L × W × H mm) — needed for access planning in confined geometries

Generator model numbers for all four machines — may provide additional technique constraints

This document was prepared by Onnex AI Agency as part of the NDT Portal v1 engagement. Supplemented specifications are sourced from publicly available manufacturer datasheets and are provided for configuration purposes only. Customer-provided data takes precedence in all cases. If any specification conflicts with the client's calibrated equipment records, the calibrated records should be used.